# Project Introduction: USB HID (Fake Keyboard) Attack Simulation for CS520 Lab

Project title: Controlled Evaluation of USB HID-Based Attack Vectors using an ESP32-C3 Keyboard Emulator

Team name: The Asian Persistent Threat

Short description:
This project studies how malicious USB Human Interface Devices (HID), specifically a “fake keyboard”, can be used to deliver automated command sequences to host systems, how defenders can observe and detect those activities on the network and host, and what practical mitigations reduce risk. The lab uses an ESP32-C3(a super mini version of a regular ESP 32) to emulate a USB keyboard that can issue automated keystrokes to target hosts in a controlled environment. A centralized command hub is used to coordinate benign, observable test actions across multiple operating systems (Windows, macOS, Linux) so we can compare how each OS behaves, what telemetry is produced, and how detection/containment differs.

## Motivation & Learning Objectives

- Demonstrate the threat model for USB HID devices that impersonate keyboards and can execute automated commands when plugged in.

- Build a reproducible, cross-platform testbed (ESP32-C3 HID emulator + centralized command hub) to compare how Windows, macOS, and Linux respond to scripted HID input.

- Capture and analyze network and host-level telemetry generated during those events.

- Evaluate and propose practical detection and mitigation strategies (endpoint policies, USB device control, user/OS prompts, host-based monitoring rules, least-privilege guidance).

- Teach safe lab practices for handling firmware/HID research and responsible disclosure procedures for discovered vulnerabilities.

## Scope (what will be done)

- Hardware HID emulator: Use an ESP32-C3 configured to present as a USB keyboard to a host. The device will send benign automation keystrokes for testing (e.g., open a terminal, run commands that write to a local test file).

- Centralized command hub (control plane): A lab-only control application to schedule and orchestrate test sequences to multiple ESP32-C3 units for repeatability and comparison across OS.

- Cross-platform test matrix: Execute the same benign sequences on Windows, macOS, and Linux VMs; record the host reactions, prompts, and telemetry.

- Traffic & telemetry capture: Capture USB attach logs, process creation events (e.g., Sysmon), and network traces between the command hub and lab devices. Analyze differences and identify signal patterns useful for detection.

## Methodology (brief)

- Prepare isolated lab environment: dedicated VLAN, snapshot able VMs, and endpoint logging enabled.

- Build/configure the ESP32-C3 to emulate USB HID for benign automation sequences. Validate on a victim VM.

- Implement the command hub to schedule and log actions; ensure all control traffic lives on the isolated lab network.

- Execute the test matrix across OSes, collect logs, and take VM snapshots before/after each run.

- Analyze collected telemetry for indicators that reliably correlate with HID-driven automation events.

- Produce final recommendations and lab report.